- Keypad interfacing

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity tb_keypad is

Port ( clk : in STD_LOGIC; -- 100 MHz clock

row : in STD_LOGIC_VECTOR (3 downto 0); -- Rows of keypad (inputs)

col : out STD_LOGIC_VECTOR (3 downto 0); -- Columns of keypad (outputs)

seg : out STD_LOGIC_VECTOR (6 downto 0); -- Seven-segment cathodes

an : out STD_LOGIC_VECTOR (3 downto 0)); -- Seven-segment anodes

end tb_keypad;

architecture Behavioral of tb_keypad is

signal col_sel : std_logic_vector(1 downto 0) := "00";

signal display_digit : std_logic_vector(3 downto 0) := "1111"; -- Latched display

signal counter: unsigned(28 downto 0) := (others => '0');

signal clk_div : std_logic := '0';

begin

an <= "1110"; -- Enable rightmost digit

-- Clock divider process

process(clk)

begin

if rising_edge(clk) then

counter <= counter + 1;

if counter = 19 then -- This value determines the scanning speed.

clk_div <= not clk_div;

counter <= (others => '0');

end if;

end if;

end process;

-- Drive columns based on col_sel

with col_sel select

col <= "0111" when "00",

"1011" when "01",

"1101" when "10",

"1110" when "11",

"1111" when others;

-- Keypad scanning and display latching process

process(clk_div)

begin

if rising_edge(clk_div) then

case col_sel is

when "00" =>

if row(3) = '0' then display_digit <= "0001"; -- 1

elsif row(2) = '0' then display_digit <= "0100"; -- 4

elsif row(1) = '0' then display_digit <= "0111"; -- 7

elsif row(0) = '0' then display_digit <= "0000"; -- 0

end if;

when "01" =>

if row(3) = '0' then display_digit <= "0010"; -- 2

elsif row(2) = '0' then display_digit <= "0101"; -- 5

elsif row(1) = '0' then display_digit <= "1000"; -- 8

elsif row(0) = '0' then display_digit <= "1111"; -- F

end if;

when "10" =>

if row(3) = '0' then display_digit <= "0011"; -- 3

elsif row(2) = '0' then display_digit <= "0110"; -- 6

elsif row(1) = '0' then display_digit <= "1001"; -- 9

elsif row(0) = '0' then display_digit <= "1110"; -- E

end if;

when "11" =>

if row(3) = '0' then display_digit <= "1010"; -- A

elsif row(2) = '0' then display_digit <= "1011"; -- B

elsif row(1) = '0' then display_digit <= "1100"; -- C

elsif row(0) = '0' then display_digit <= "1101"; -- D

end if;

when others =>

null;

end case;

col_sel <= std_logic_vector(unsigned(col_sel) + 1); -- Cycle col_sel to scan all columns

end if;

end process;

-- Seven-segment decoder

process(display_digit)

begin

case display_digit is

when "0000" => seg <= "1000000"; -- 0

when "0001" => seg <= "1111001"; -- 1

when "0010" => seg <= "0100100"; -- 2

when "0011" => seg <= "0110000"; -- 3

when "0100" => seg <= "0011001"; -- 4

when "0101" => seg <= "0010010"; -- 5

when "0110" => seg <= "0000010"; -- 6

when "0111" => seg <= "1111000"; -- 7

when "1000" => seg <= "0000000"; -- 8

when "1001" => seg <= "0010000"; -- 9

when "1010" => seg <= "0001000"; -- A

when "1011" => seg <= "0000011"; -- B

when "1100" => seg <= "1000110"; -- C

when "1101" => seg <= "0100001"; -- D

when "1110" => seg <= "0000110"; -- E

when "1111" => seg <= "0001110"; -- F

when others => seg <= "1111111"; -- Blank

end case;

end process;

end Behavioral;

XDC FILE

set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {col[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {row[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {seg[0]}]

set_property PACKAGE_PIN W4 [get_ports {an[3]}]

set_property PACKAGE_PIN V4 [get_ports {an[2]}]

set_property PACKAGE_PIN U4 [get_ports {an[1]}]

set_property PACKAGE_PIN U2 [get_ports {an[0]}]

set_property PACKAGE_PIN G2 [get_ports {col[3]}]

set_property PACKAGE_PIN J2 [get_ports {col[2]}]

set_property PACKAGE_PIN L2 [get_ports {col[1]}]

set_property PACKAGE_PIN J1 [get_ports {col[0]}]

set_property PACKAGE_PIN G3 [get_ports {row[3]}]

set_property PACKAGE_PIN H2 [get_ports {row[2]}]

set_property PACKAGE_PIN K2 [get_ports {row[1]}]

set_property PACKAGE_PIN H1 [get_ports {row[0]}]

set_property PACKAGE_PIN U7 [get_ports {seg[6]}]

set_property PACKAGE_PIN V5 [get_ports {seg[5]}]

set_property PACKAGE_PIN U5 [get_ports {seg[4]}]

set_property PACKAGE_PIN V8 [get_ports {seg[3]}]

set_property PACKAGE_PIN U8 [get_ports {seg[2]}]

set_property PACKAGE_PIN W6 [get_ports {seg[1]}]

set_property PACKAGE_PIN W7 [get_ports {seg[0]}]

set_property PACKAGE_PIN W5 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

- COUNTER

counter vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity mod25 is

Port (

rst     : in  STD_LOGIC;

pr      : in  STD_LOGIC;

clk     : in  STD_LOGIC;

dir     : in  STD_LOGIC;

Q       : out STD_LOGIC_VECTOR (4 downto 0);

clk_div : out STD_LOGIC               -- Added output

);

end mod25;

architecture mod25_arch of mod25 is

signal Qtemp   : STD_LOGIC_VECTOR (4 downto 0) := "00000";

signal counter : STD_LOGIC_VECTOR (30 downto 0) := (others => '0');

signal clk_div_int : STD_LOGIC;

begin

-- Clock divider

process(clk)

begin

if rising_edge(clk) then

counter <= counter + 1;

end if;

end process;

clk_div_int <= counter(25);  -- Internal divided clock

clk_div <= clk_div_int;      -- Output it to testbench

-- MOD-25 counter

process(rst, pr, clk_div_int, dir)

begin

if rst = '1' then

Qtemp <= (others => '0');

elsif pr = '1' then

Qtemp <= (others => '1');

elsif falling_edge(clk_div_int) then

if dir = '1' then

if Qtemp < "11000" then    -- 24 decimal

Qtemp <= Qtemp + 1;

else

Qtemp <= (others => '0');

end if;

else

if Qtemp > "00111" then    -- 7 decimal

Qtemp <= Qtemp - 1;

else

Qtemp <= (others => '1');

end if;

end if;

end if;

end process;

Q <= Qtemp;

end mod25_arch;

tb

library ieee;

use ieee.std_logic_1164.all;

entity mod25_tb is

end mod25_tb;

architecture behavior of mod25_tb is

-- Component Declaration for the Unit Under Test (UUT)

component mod25

port(

rst     : in  std_logic;

pr      : in  std_logic;

clk     : in  std_logic;

dir     : in  std_logic;

Q       : out std_logic_vector(4 downto 0);

clk_div : out std_logic        -- Must match entity!

);

end component;

-- Inputs

signal rst : std_logic := '0';

signal pr  : std_logic := '0';

signal clk : std_logic := '0';

signal dir : std_logic := '0';

-- Outputs

signal Q       : std_logic_vector(4 downto 0);

signal clk_div : std_logic;

-- Clock period

constant clk_period : time := 10 ns;

begin

-- Instantiate UUT

uut: mod25 port map (

rst     => rst,

pr      => pr,

clk     => clk,

dir     => dir,

Q       => Q,

clk_div => clk_div

);

----------------------------------------------------------------

-- Clock generation

----------------------------------------------------------------

clk_process: process

begin

clk <= '0';

wait for clk_period / 2;

clk <= '1';

wait for clk_period / 2;

end process;

----------------------------------------------------------------

-- Stimulus

----------------------------------------------------------------

stim_proc_dir: process

begin

dir <= not dir;

wait for 320 ns;

end process;

stim_proc_rst: process

begin

wait for 680 ns;

rst <= '1';

wait for 40 ns;

rst <= '0';

wait;

end process;

stim_proc_pr: process

begin

wait for 750 ns;

pr <= '1';

wait for 40 ns;

pr <= '0';

wait;

end process;

end behavior;

xdc

set_property IOSTANDARD LVCMOS33 [get_ports {Q[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {Q[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk_div]

set_property IOSTANDARD LVCMOS33 [get_ports pr]

set_property IOSTANDARD LVCMOS33 [get_ports dir]

set_property IOSTANDARD LVCMOS33 [get_ports rst]

set_property PACKAGE_PIN W5 [get_ports clk]

set_property PACKAGE_PIN U16 [get_ports clk_div]

set_property PACKAGE_PIN V16 [get_ports dir]

set_property PACKAGE_PIN W16 [get_ports pr]

set_property PACKAGE_PIN W17 [get_ports rst]

set_property PACKAGE_PIN L1 [get_ports {Q[4]}]

set_property PACKAGE_PIN P1 [get_ports {Q[3]}]

set_property PACKAGE_PIN N3 [get_ports {Q[2]}]

set_property PACKAGE_PIN P3 [get_ports {Q[1]}]

set_property PACKAGE_PIN U3 [get_ports {Q[0]}]

set_property CLOCK_DEDICATED_ROUTE FALSE [get_nets pr]

3)FIFO

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity fifo_correct is

GENERIC

(

ADDRESS_WIDTH : integer:=2;---8 bit

DATA_WIDTH : integer:=4 ---32 bit

);

port ( clk : in std_logic;

clk_div : inout std_logic;

reset : in std_logic;

enr : in std_logic; --enable read,should be '0' when not in use.

enw : in std_logic; --enable write,should be '0' when not in use.

dataout : out std_logic_vector(DATA_WIDTH-1 downto 0); --output data

datain : in std_logic_vector (DATA_WIDTH-1 downto 0); --input data

empty : out std_logic; --set as '1' when the queue is empty

err : out std_logic;

full : out std_logic --set as '1' when the queue is full

);

end fifo_correct;

architecture Behavioral of fifo_correct is

type memory_type is array (0 to ((2**ADDRESS_WIDTH)-1)) of std_logic_vector(DATA_WIDTH-1 downto

0); -----distributed-------

signal memory : memory_type ;-- :=(others => (others => '0')); --memory for queue.-----

signal readptr,writeptr : std_logic_vector(ADDRESS_WIDTH-1 downto 0); --read and write pointers.

signal full0 : std_logic;

signal empty0 : std_logic;

signal counter: std_logic_vector(28 downto 0):=( others=>'0');

begin

full <= full0;

empty <= empty0;

fifo0: process(clk_div,reset,datain,enw,enr)

begin

if reset='1' then

readptr <= (others => '0');

writeptr <= (others => '0');

empty0 <='1';

full0<='0';

err<='0';

elsif  clk_div'event and clk_div = '1' then

if enw='1' and full0='0' then

memory (conv_integer(writeptr)) <= datain ;

writeptr <= writeptr + '1' ;

if (writeptr + '1' = readptr) then

full0<='1';

empty0<= '0';

else

full0<='0';

empty0<= '1';

end if ;

end if ;

if enr='1' and empty0='0' then

dataout <= memory (conv_integer(readptr));

readptr <= readptr + '1' ;

if (readptr + '1'  = writeptr ) then

empty0<='1';

full0<='0';

else

empty0<='0';

full0<='1';

end if ;

end if ;

if (empty0='1' and enr='1') or (full0='1' and enw='1') then

err<='1';

else

err<= '0';

eAnd if ;

end if;

end process;

process(clk)

begin

if clk'event and clk= '1' then

counter<= counter + '1';

end if;

end process;

clk_div<= counter(26);

end Behavioral;

TB

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- Uncomment the following library declaration if using -- arithmetic functions with Signed or Unsigned values --USE ieee.numeric_std.ALL;

ENTITY fifo_test IS

END fifo_test;

ARCHITECTURE behavior OF fifo_test IS

-- Component Declaration for the Unit Under Test (UUT)

component fifo_correct is

GENERIC

(

ADDRESS_WIDTH : integer:=2;---8 bit

DATA_WIDTH : integer:=4 ---32 bit

);

port ( clk : in std_logic;

clk_div : inout std_logic;

reset : in std_logic;

enr : in std_logic; --enable read,should be '0' when not in use.

enw : in std_logic; --enable write,should be '0' when not in use.

dataout : out std_logic_vector(DATA_WIDTH-1 downto 0); --output data

datain : in std_logic_vector (DATA_WIDTH-1 downto 0); --input data

empty : out std_logic; --set as '1' when the queue is empty

err : out std_logic;

full : out std_logic --set as '1' when the queue is full

);

end component;

--Inputs

signal clk,clk_div,reset : std_logic := '0';

signal enr : std_logic := '0';

signal enw : std_logic := '0';

signal datain : std_logic_vector(3 downto 0) := (others => '0');

--Outputs

signal dataout : std_logic_vector(3 downto 0);

signal empty : std_logic:='0';

signal err : std_logic:='0';

signal full : std_logic:='0';

-- Clock period definitions

constant clk_period : time := 10 ns;

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: fifo_correct PORT MAP (

clk => clk,

clk_div => clk_div,

reset => reset,

enr => enr,

enw => enw,

dataout => dataout,

datain => datain,

empty => empty,

err => err,

full => full

);

-- Clock process definitions

clk_process :process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

reset<= '1','0' after 20 ns;

enw<= '1', '0' after 600 ns ;

enr<= '0','1' after  600 ns ;

--Stimulus process

stim_proc: process

begin

datain<="1010";

wait for 20 ns;

datain<="1111";

wait for 20 ns;

datain<="1001";

wait for 20 ns;

datain<="0001";

wait for 20 ns;

end process;

END;

4. ALU

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

use IEEE.std_logic_unsigned.all;

entity ALU is

Port ( A : in STD_LOGIC_VECTOR (3 downto 0);

B : in STD_LOGIC_VECTOR (3 downto 0);

S : in STD_LOGIC_VECTOR (2 downto 0);

Y : out STD_LOGIC_VECTOR (3 downto 0));

end ALU;

architecture Behavioral of ALU is

begin

with S select

Y <= A + B when "000",

A - B when "001",

A + 1 when "010",

B + 1 when "011",

A and B when "100",

A or B when "101",

A nand B when "110",

A nor B when "111",

A xor B when others;

end Behavioral;

TB

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

use IEEE.std_logic_unsigned.all;

Entity ALU_TB is

end ALU_TB;

architecture Behavioral of ALU_TB is

component ALU

Port ( A : in STD_LOGIC_VECTOR (3 downto 0);

B : in STD_LOGIC_VECTOR (3 downto 0);

S : in STD_LOGIC_VECTOR (2 downto 0);

Y : out STD_LOGIC_VECTOR (3 downto 0));

end component;

signal A,B,Y:std_logic_vector (3 downto 0);

signal S:std_logic_vector (2 downto 0):=(others => '0');

begin

U:ALU port map (A,B,S,Y);

Process

begin

A <= "0011";

B <= "1100";

S <= "000";

wait for 100 ns;

S <= "001";

wait for 100 ns;

S <= "010";

wait for 100 ns;

S <= "011";

wait for 100 ns;

S <= "100";

wait for 100 ns;

S <= "101";

wait for 100 ns;

S <= "110";

wait for 100 ns;

S <= "111";

wait for 100 ns;

end process;

end Behavioral;

5. USR

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity usr53 is

Port (

clk : in  STD_LOGIC;

clk_div : inout std_logic;

rst : in  STD_LOGIC;

load: in  STD_LOGIC;

mode : in  STD_LOGIC_VECTOR(1 downto 0);  -- mode selection input

si : in  STD_LOGIC;  -- serial input

pi : in  STD_LOGIC_VECTOR(3 downto 0);  -- parallel input

so : out  STD_LOGIC;  -- serial output

po : out  STD_LOGIC_VECTOR(3 downto 0)  -- parallel output

);

end usr53;

architecture Behavioral of usr53 is

signal shift_reg : STD_LOGIC_VECTOR(3 downto 0);

signal counter: std_logic_vector(26 downto 0):=( others=>'0');

begin

process(clk_div, rst,load)

begin

if rst = '1' then

shift_reg <= (others => '0');

elsif clk_div'event and clk_div= '1' then

case mode is

when "00" =>  -- SISO mode

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0)<=si;

so <= shift_reg(3);

when "01" =>  -- SIPO mode

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

shift_reg(0)<=si;

po <= shift_reg;

when "10" =>  -- PISO mode

if(load='0') then

shift_reg <= pi;

else

shift_reg( 3 downto 1) <= shift_reg(2 downto 0);

end if;

so <= shift_reg(3);

when "11" =>  -- PIPO mode

po<= pi;

when others =>

null;

end case;

end if;

end process;

-- so <= shift_reg(3);

-- po <= shift_reg;

process(clk)

begin

if clk'event and clk = '1' then

if rst = '1' then

counter <= (others => '0');

else

counter <= counter + '1';

end if;

end if;

end process;

clk_div<= counter(25);

end Behavioral;

TB

Library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity usr1tb is

end usr1tb;

architecture Behavioral of usr1tb is

signal clk,clk_div : STD_LOGIC := '0';

signal rst : STD_LOGIC := '0';

signal load : STD_LOGIC := '0';

signal mode : STD_LOGIC_VECTOR(1 downto 0) := "00";

signal si : STD_LOGIC := '0';

signal pi : STD_LOGIC_VECTOR(3 downto 0) := (others => '0');

signal so : STD_LOGIC;

signal po : STD_LOGIC_VECTOR(3 downto 0);

component  usr53

Port (

clk : in  STD_LOGIC;

clk_div : inout std_logic;

rst : in  STD_LOGIC;

load: in  STD_LOGIC;

mode : in  STD_LOGIC_VECTOR(1 downto 0);

si : in  STD_LOGIC;

pi : in  STD_LOGIC_VECTOR(3 downto 0);

so : out  STD_LOGIC;

po : out  STD_LOGIC_VECTOR(3 downto 0)

);

end component;

begin

uut : usr53

Port Map (

clk => clk,

clk_div => clk_div,

rst => rst,

load => load,

mode => mode,

si => si,

pi => pi,

so => so,

po => po

);

clk_process : process

begin

clk <= '0';

wait for 10 ns;

clk <= '1';

wait for 10 ns;

end process;

stim_proc : process

begin

-- Reset the UUT

rst <= '1';

wait for 20 ns;

rst <= '0';

-- Test SISO mode

mode <= "00";

si <= '1';

wait for 300 ns;

si <= '0';

wait for 400 ns;

-- Test SIPO mode

mode <= "01";

si <= '1';

wait for 200 ns;

si <= '0';

wait for 500 ns;

-- Test PISO mode

mode <= "10";

load<='0';

pi <= "1110";

wait for 100 ns;

load<='1';

wait for 300 ns;

-- Test PIPO mode

mode <= "11";

pi <= "1010";

wait for 50 ns;

pi <= "1100";

wait for 50 ns;

wait;

end process;

end Behavioral;